WIRELESS M-BUS
DATA LINK USER GUIDE

WISE SDK / protocol/wmbus_datalink

Formal User and Integration Manual

Revision 1.2  |  2026-09-01

| Document Item | Value |
| --- | --- |
| Product / Module | Wireless M-Bus Data Link |
| Primary Audience | Firmware engineers, integration engineers, validation engineers, and technical support |
| Primary Roles | Gateway (GW) and Meter (MTR) |
| Security Modes Covered | 0, 5, and 7 |
| Primary PHY Example | T2 |

| IMPORTANT | This manual consolidates the supplied implementation and feature documentation. Current implementation behavior is described in the operational and feature sections below. |
| --- | --- |

# Document Control

| Revision | Date | Description |
| --- | --- | --- |
| 1.0 | 2026-09-01 | Initial consolidated English user guide based on supplied WMBus design, feature, packet-format, FSM, radio-test, and log-parser materials. |
| 1.1 | 2026-09-01 | Updated TOC navigation, timing-sequence figures, whitelist deployment guidance, project usage guidance, and source/reference presentation. |
| 1.2 | 2026-09-01 | Added build-time diagnostics and timing instrumentation definitions, constraints, and recommended build combinations. |

## How to Use This Manual

The manual is organized in the order a typical engineering user needs information: orientation and prerequisites, packet and state-machine concepts, configuration, normal operation, advanced timing architecture, test tools, diagnostics, and reference material.

# Table of Contents

1. Introduction	3

1.1 Purpose	3

1.2 Intended Audience	4

1.3 Scope	4

1.4 Conventions	4

2. Product Overview	4

2.1 Functional Model	4

2.2 Supported Operating Modes	4

2.3 T2 Timing Context	5

2.4 Normal Datalink vs. Frame-A Radio Test	5

3. Packet Organization	5

3.1 Layer View	5

3.2 Frame A First Block	5

3.3 Short Transport Layer (STL) Example	6

3.4 Long Transport Layer (LTL) Example	6

3.5 Optional Blocks	6

3.6 Addressing Note	6

4. Current State-Machine Model	7

4.1 Timing Terms	7

4.2 Gateway-Side States	7

4.3 Meter-Side Session States	7

4.4 Meter Installation Flow	8

4.5 Retry and FCB Behavior	8

5. Getting Started	8

5.1 Typical Two-Device Setup	8

5.2 Recommended Configuration Order	8

6. Security Mode Configuration	8

6.1 Supported Modes	8

6.2 Provisioned, Not Negotiated	9

6.3 Empty Control Frames	9

6.4 Meter Commands	9

6.5 Key Configuration	9

7. Gateway Whitelist and Admission Control	9

7.1 Whitelist Entry	9

7.2 Fixed and Dynamic Policies	9

7.3 Temporary CLI Initialization	10

7.4 Add, Delete, Clear, Free, and List	10

7.5 Admission Control	10

7.6 Safe Operation Rule	10

8. Normal GW/Meter Operation	10

8.1 Mode 0 Verification	11

8.2 Mode 5 Verification	11

8.3 Mode 7 Verification	11

8.4 Application Queue Behavior	11

9. Gateway Low-Latency TX Architecture	12

9.1 Design Objective	12

9.2 Prepared and Pre-Packed Frames	12

9.3 Per-Connection TX Ownership	12

9.4 Late DLL First-Block Finalization	12

9.5 Early-RX	12

9.6 Commit and Fallback	13

9.7 TX_DONE, Confirmation, and Retry	13

10. Frame-A Radio Test Mode	13

10.1 Purpose and Boundary	13

10.2 Build Configuration	13

10.3 Commands	14

10.4 Role and Direction Rules	14

10.5 Test Frame	14

10.6 Standard T2 Two-Device Procedure	14

11. WMBus Log Parser	15

11.1 Purpose	15

11.2 Basic Usage	15

11.3 Important Options	15

11.4 Diagnostic Codes	15

11.5 Result Classification	16

11.6 Expected Tagged Events	16

12. Troubleshooting	16

13. CLI Quick Reference	17

14. Integration Reference	17

14.1 Principal Whitelist Implementation Points	17

14.2 Principal Low-Latency TX Implementation Points	17

14.3 Event / Platform Integration	18

14.4 Admission Result Callback	18

15. Operational Rules and Limitations	18

16. Glossary	18

17. Supplied Project References	19

17.1 External Structural Reference	19

Appendix A. Verification Checklist	19

Appendix B. Example Log Analysis Command	20

# 1. Introduction

## 1.1 Purpose

This guide explains how to configure, operate, verify, and diagnose the Wireless M-Bus data-link implementation in WISE SDK. It focuses on the implemented Gateway and Meter behavior, including packet organization, session control, Security Modes 0/5/7, Gateway whitelist policy, the low-latency pre-encryption/pre-packed TX and Early-RX architecture, the dedicated Frame-A radio test, and the WMBus UART log parser.

## 1.2 Intended Audience

- Firmware engineers integrating protocol/wmbus_datalink.

- Validation engineers running two-device GW/Meter tests.

- RF engineers using the dedicated Frame-A radio test configuration.

- Support engineers interpreting WMBus UART logs and session failures.

- Application developers provisioning Gateway whitelist and Security Mode policy.

## 1.3 Scope

The guide documents behavior supported by the supplied implementation materials. It does not attempt to reproduce the complete EN 13757 or OMS specifications. General protocol background is included only where it helps operate the implementation.

## 1.4 Conventions

| Convention | Meaning |
| --- | --- |
| GW | Gateway / Other-side WMBus role in the normal datalink product. |
| MTR | Meter role. |
| APP | Application layer or application-owned configuration. |
| DLL / ELL / AFL / TPL / APL | Protocol-layer abbreviations used by the implementation and packet-format material. |
| CLI | Shell commands beginning with wm. |
| IMPORTANT | A rule that can affect correctness, security policy, memory ownership, or session behavior. |
| NOTE | Supporting explanation or operational clarification. |

# 2. Product Overview

## 2.1 Functional Model

The implementation provides a WMBus data-link service between a Meter and a Gateway. The normal product path combines the application, datalink state machine, packet parsing/construction, security processing, and the WMBus RF driver. The general project material identifies the main logical elements as main, wmbus_api, wmbus_fsm, and radio_driver.

| Area | Primary Responsibility |
| --- | --- |
| Application / CLI | Configure role, PHY mode, device identity, security policy, whitelist, payloads, and diagnostics. |
| Datalink API | Packet creation/parsing, connection and whitelist handling, per-device policy, and application data flow. |
| FSM | Controls Meter/Gateway session progression, response/retry behavior, and timing-driven transitions. |
| Security | Applies configured Security Mode to application payloads and validates received payload policy. |
| RF interface | WMBus framing, Frame A CRC, line coding, RX/TX control, and packed-frame staging. |
| Diagnostic tooling | Tagged UART logging plus offline parser for event, error, session, and cross-source analysis. |

## 2.2 Supported Operating Modes

| Mode Value | WMBus Mode | Direction Class |
| --- | --- | --- |
| 0 | S1 | Unidirectional |
| 1 | S2 | Bidirectional |
| 2 | T1 | Unidirectional |
| 3 | T2 | Bidirectional |
| 4 | C1 | Unidirectional |
| 5 | C2 | Bidirectional |

| NOTE | The dedicated Frame-A radio test does not expose R2 because the current RF driver does not support it. |
| --- | --- |

## 2.3 T2 Timing Context

The supplied general introduction identifies Mode T2 response timing with tRO(min) = 2 ms and tRO(max) = 3 ms, giving a 1 ms receive window. The current Gateway architecture therefore moves expensive packet construction, security processing, and RF packing out of the RX_DONE-to-TX critical path.

## 2.4 Normal Datalink vs. Frame-A Radio Test

| Normal Datalink | Frame-A Radio Test |
| --- | --- |
| Uses the normal WMBus FSM and session behavior. | Normal datalink FSM is not started. |
| Uses security, whitelist, connections, queues, and application payload flow. | Does not use security, whitelist, admission control, connection tables, sessions, pre-encryption, pre-packed datalink buffers, or Early-RX. |
| Tests complete product-level link behavior. | Isolates RF-driver and WMBus Frame A transmit/receive behavior. |

# 3. Packet Organization

## 3.1 Layer View

The supplied packet-format worksheet organizes the application frame into DLL, optional ELL, AFL, TPL (STL), and APL information. Security-related MAC and encryption regions span the transport/application content according to the selected security profile.

| Layer | Fields Shown in the Packet Reference |
| --- | --- |
| DLL | L Field, C Field, Manufacturer field, Address / ID, Version, Device Type. |
| ELL | CI Field, CC Field, Access Number. |
| AFL | CI Field, AFLL, FCL, MCL, KI, MCR, MAC. |
| TPL (STL) | ML, CI Field, Access Number, Status, Configuration Field, CFE, AES-Verify. |
| APL | Payload and padding. |

## 3.2 Frame A First Block

| Field | Size |
| --- | --- |
| L-field | 1 byte |
| C-field | 1 byte |
| M-field | 2 bytes |
| A-field | 6 bytes |
| CRC | 2 bytes |

## 3.3 Short Transport Layer (STL) Example

For the STL layout shown with CI = 0x7A, the second block contains the following fields:

| Field | Size |
| --- | --- |
| CI-field | 1 byte |
| ACC-field | 1 byte |
| STS-field | 1 byte |
| Configuration field | 2 bytes |
| Decrypt Verify | 2 bytes |
| Payload | 16*N - 2 bytes |
| CRC | 2 bytes |

## 3.4 Long Transport Layer (LTL) Example

For the LTL layout shown with CI = 0x5B, the second block additionally carries the long meter identity information:

| Field | Size |
| --- | --- |
| CI-field | 1 byte |
| Identification number | 4 bytes |
| Manufacturer | 2 bytes |
| Version | 1 byte |
| Device Type | 1 byte |
| ACC-field | 1 byte |
| STS-field | 1 byte |
| Configuration field | 2 bytes |
| Decrypt Verify | 2 bytes |
| Payload | 16*N - 2 bytes |
| CRC | 2 bytes |

## 3.5 Optional Blocks

Additional Frame A blocks carry payload plus a 2-byte CRC. The worksheet describes a 16-byte payload block, with the last block shortened according to the remaining frame length.

## 3.6 Addressing Note

The early design material distinguishes Link Layer Address from Meter Application Address. It also records the rule that the long TPL header contains the meter address regardless of transmission direction and is required when the Link Layer Address differs from the meter address.

Figure 1. Supplied packet-format reference (PKT format worksheet).

# 4. Current State-Machine Model

## 4.1 Timing Terms

| Term | Meaning in the FSM |
| --- | --- |
| tRO | Response delay for Other-to-Meter communication. |
| tRM | Response delay for Meter-to-Other communication. |
| tTxD | FAC transmission delay. |
| tTO | FAC timeout. |
| tDEC | Meter decision point. |
| tRO_min / tRO_max | Meter receive-window boundaries. |

## 4.2 Gateway-Side States

| State | Representative Behavior |
| --- | --- |
| STATE_STANDBY | Wait for RX activity / new session traffic. |
| W4_PKT_TX | Wait for a selected GW packet to complete transmission. |
| W4_RSP_RX | Wait for the Meter response; timeout and response handling control the next request, ACK, or SND-NKE path. |

The GW flow uses RX completion, TX completion, response timeout, data-request conditions, and access-demand information to decide whether to continue the exchange, transmit another request/data packet, acknowledge, or terminate with SND-NKE.

## 4.3 Meter-Side Session States

| State | Representative Behavior |
| --- | --- |
| SUBSTATE_MTR_STANDBY | Idle Meter session state before a trigger/decision point starts transmission. |
| W4_FRT_PKT_TX | Wait for the first Meter packet transmission to complete. |
| W4_REQ_PKT_RX | Wait for the Gateway request/response after the first Meter packet. |
| W4_RSP_ACK_TX | Transmit the Meter response/acknowledgement and arm the next receive timing. |
| W4_NEXT_PKT_RX | Wait for the next Gateway packet in the active session. |

## 4.4 Meter Installation Flow

| State | Purpose |
| --- | --- |
| SUBSTATE_IDLE | Installation idle state. |
| W4_SND_IR_TX | Transmit installation request and arm installation response timing. |
| W4_CNF_IR_RX | Wait for installation confirmation; retry on receive timeout according to the FSM. |

The supplied FSM notes a random installation delay of 30-60 seconds and a random NKE delay of 5-25 seconds.

## 4.5 Retry and FCB Behavior

The implementation treats retries as continuation of the same transaction. A retry must retain the in-flight frame and its transmitted FCB. FCB advances only after the expected Meter acknowledgement/response for that transaction is accepted. This rule is also enforced by the current pre-packed TX architecture so that payload, ciphertext, MAC, access information, and DLL state remain consistent.

# 5. Getting Started

## 5.1 Typical Two-Device Setup

A normal verification setup uses one Gateway and one Meter. The feature verification procedures use T2 and the following example identities:

| Role | Example Device ID |
| --- | --- |
| Gateway | 0x80000002 |
| Meter | 0x80000001 |

## 5.2 Recommended Configuration Order

- Configure debug level and role.

- Select the WMBus PHY mode.

- Configure the local device information.

- On the Gateway, initialize/load the whitelist and provision the target Meter and Security Mode.

- On the Meter, configure the local Security Mode and, when required, the accepted Gateway.

- Queue optional application payloads.

- Start the WMBus link.

- Trigger the Meter session and verify logs/results.

| IMPORTANT | Whitelist structural configuration must be completed before wm start / wmbus_link_start(). A reset is required before structural whitelist reconfiguration after start. |
| --- | --- |

# 6. Security Mode Configuration

## 6.1 Supported Modes

| Mode | Application Payload Protection | Provisioning Rule |
| --- | --- | --- |
| 0 | No application-payload encryption or cryptographic authentication. | Must be explicitly configured. |
| 5 | Mode 5 AES encryption path. | Must be explicitly configured when not using Mode 7 default. |
| 7 | Mode 7 key derivation, encryption, and MAC generation. | Default when an optional GW whitelist security_mode is omitted. |

## 6.2 Provisioned, Not Negotiated

Security Mode is provisioned per device; it is not negotiated or learned over the air. The Meter uses its local setting. The Gateway stores the policy in each Meter whitelist entry. A received application payload is accepted only when the observed mode matches the configured policy. A mismatch is dropped and must not cause either side to change its configured mode.

## 6.3 Empty Control Frames

Empty ACK, SND-NKE, REQ-UD2, and other frames without application payload are link-control frames. The packet builder uses Mode 0 semantics for these empty frames even when the associated Meter application policy is Mode 5 or 7. This does not represent an application security downgrade.

## 6.4 Meter Commands

wm sec 0
wm sec 5
wm sec 7

## 6.5 Key Configuration

Mode 5 and Mode 7 require identical shared key material on the Gateway and Meter. The supplied test firmware uses the following built-in laboratory key:

00 11 22 33 44 55 66 77 88 99 aa bb cc dd ee ff

To override the test key, apply the same key on both devices before communication:

wm key <16 hexadecimal bytes>

| IMPORTANT | The built-in key is for laboratory verification. Production systems must provision application-specific key material. |
| --- | --- |

# 7. Gateway Whitelist and Admission Control

## 7.1 Whitelist Entry

Each entry stores Meter identity, configured Security Mode, and per-Meter link-layer runtime state. Current membership lookup uses the Meter device ID; manufacturer, version, and device type are stored as entry information but are not part of the membership comparison.

## 7.2 Fixed and Dynamic Policies

Recommended production integration: use the APP fixed whitelist. The customer application should provide the approved Meter whitelist and the required writable runtime-state memory at the customer-selected memory location. This keeps whitelist ownership and memory placement under application control. The APP dynamic and temporary fixed/dynamic alternatives are intended primarily for demonstration, development, enrollment experiments, or debugging and are not the recommended production configuration.

| Configuration | Storage | Unknown Meter | Default / Learned Security |
| --- | --- | --- | --- |
| APP fixed | APP device table + writable runtime-state buffer | Reject | Provisioned; compatibility API assigns Mode 7 |
| APP dynamic | APP-provided writable RAM buffer | Learn while capacity is available | Mode 7 |
| Temporary fixed | Heap-based writable test buffer | Reject by default | Provisioned |
| Temporary dynamic | Heap-based writable test buffer | Learn while capacity is available | Mode 7 |

Operational recommendation: APP fixed is the preferred customer deployment model. Other whitelist models are provided for demo/debug or controlled development use unless the product design explicitly requires dynamic enrollment.

## 7.3 Temporary CLI Initialization

wm wl init fix [capacity]
wm wl init dyn [capacity]

## 7.4 Add, Delete, Clear, Free, and List

wm wl add [id] [manufacturer] [version] [device_type] [security_mode]
wm wl del [id]
wm wl clear
wm wl free
wm wl list

If security_mode is omitted by an interface that supports an optional mode, Mode 7 is assigned.

## 7.5 Admission Control

wm adm
wm adm 0
wm adm 1

| admission_ctrl | Unknown Meter Behavior |
| --- | --- |
| 1 | Reject unknown Meter. |
| 0 | Attempt to learn an unknown Meter into writable storage, subject to capacity and storage availability. |

Dynamically learned entries are assigned Security Mode 7. Mode 0 and Mode 5 Meters therefore require explicit provisioning before the Gateway starts.

## 7.6 Safe Operation Rule

| IMPORTANT | Initialization/load, manual add, delete, clear, and free must be completed before the Gateway starts. After start, wm wl list remains allowed, and controlled internal dynamic learning may continue. wm stop does not authorize structural reconfiguration; reset is required. |
| --- | --- |

# 8. Normal GW/Meter Operation

This chapter is a command-driven verification procedure. Use the SOC_9006 project when executing the CLI sequences in Sections 8.1-8.3. To observe a complete session in which the Gateway and Meter interact as two dedicated endpoints, use the SOC_9006_GW and SOC_9006_MTR projects respectively.

## 8.1 Mode 0 Verification

Gateway:

wm dbg 4
wm role 0
wm mode 3
wm setinfo 80000002 ESM 1 7
wm wl init fix 1
wm wl add 80000001 ESM 1 7 0
wm wl list
wm start

Meter:

wm dbg 4
wm role 1
wm mode 3
wm sec 0
wm setinfo 80000001 ESM 1 7
wm gwonly 1 80000002
wm wr 5 1 2 3 4 5
wm start

Optional Gateway-to-Meter payloads and trigger:

wm wr 80000001 3 1 2 3
wm wr 80000001 4 1 2 3 4
wm fsm 0

## 8.2 Mode 5 Verification

Use the same sequence, with Mode 5 provisioned on both sides:

GW:  wm wl add 80000001 ESM 1 7 5
MTR: wm sec 5

Expected behavior includes Mode 5 in both application directions, first downlink preparation before the Meter request, and no security mismatch or encryption error.

## 8.3 Mode 7 Verification

GW:  wm wl add 80000001 ESM 1 7 7
MTR: wm sec 7

Omitting the final whitelist security-mode argument is also defined to select Mode 7. Expected behavior includes successful encryption/MAC processing and ordered delivery of queued Gateway payloads.

## 8.4 Application Queue Behavior

When the Gateway is in standby and the target queue was empty, the first application payload can be prepared immediately. Additional payloads are queued. After the current frame is transmitted, the next payload is prepared before it is needed. If an application write occurs during an active session, the payload remains queued and is prepared after the connection returns to standby for the next session.

# 9. Gateway Low-Latency TX Architecture

## 9.1 Design Objective

The Gateway minimizes RX-to-TX turnaround by completing packet construction, security processing, RF frame packing, and FCB first-block preparation before the final response is required. After the Meter frame is validated, the critical path is reduced to prepared-frame selection/promotion, current DLL/FCB finalization, packed-frame staging, RF configuration, and TX start.

## 9.2 Prepared and Pre-Packed Frames

| Security Mode | Advance Processing |
| --- | --- |
| 0 | DLL/TPL construction with plaintext application payload; RF frame is pre-packed. |
| 5 | DLL/TPL construction plus Mode 5 AES encryption; RF frame is pre-packed. |
| 7 | DLL/ELL/AFL/TPL construction, key derivation, encryption, and MAC; RF frame is pre-packed. |

Timing sequence - pre-encrypted data with a pre-packed frame. The packet and RF-packed data are already prepared before the receive-complete interrupt. After RX ISR, the TX path mainly enters the TX API, configures the TX/radio frame, and starts transmission.

Figure 2. Pre-encryption DATA with pre-packed timing sequence.

## 9.3 Per-Connection TX Ownership

| Buffer | Purpose |
| --- | --- |
| tx_next_packed_buffer | Next prepared frame not yet selected for TX. |
| tx_in_flight_packed_buffer | Selected frame whose transaction result is still pending. |
| tx_nke_packed_buffer | Dedicated prepared SND-NKE control frame. |

Promotion swaps buffer ownership and metadata rather than copying and repacking the full frame.

## 9.4 Late DLL First-Block Finalization

The Gateway prebuilds encoded first-block variants for FCB=0 and FCB=1. Immediately before TX it applies the current link-layer semantics. A retry retains the FCB already used by the in-flight frame; a new transaction uses the current connection FCB. Only the packed first block is replaced, so encrypted payload, MAC, later CRC blocks, and remaining line-coded data are not rebuilt.

## 9.5 Early-RX

During GW T2 reception, Early-RX samples the Meter DLL header before RX_DONE. The default sample delay is 1800 microseconds; the documented CLI range is 100 to 10000 microseconds in 100-microsecond steps.

wm earlyrx delay [us]

Candidate selection order:

1.  Valid in-flight application-data frame for the Meter.

2.  Valid next prepared frame for the Meter.

3.  First-session empty REQ-UD2 candidate for a known Meter without an existing connection.

Timing sequence - Early-RX pre-packed candidate. After SYNC ISR, the DLL header is sampled early and the candidate preparation/packing work can be performed while reception is still in progress. After RX ISR completes the normal validation path, the prepared candidate proceeds through the TX API, TX/radio-frame configuration, and TX start.

Figure 3. Early-RX pre-packed candidate timing sequence.

| IMPORTANT | An Early-RX candidate is never transmitted solely from the sampled header. The complete frame must pass normal frame, parsing, whitelist/admission, connection/state, security, decryption/authentication, and FSM validation before commit. |
| --- | --- |

## 9.6 Commit and Fallback

Commit verifies that the Early-RX generation, DLL header, Meter identity, connection lifetime, packed-buffer identity/length/type, and prepared snapshot still match the fully received transaction. If any condition fails or no candidate exists, the Gateway uses the normal prepared-frame fallback path.

## 9.7 TX_DONE, Confirmation, and Retry

- After GW TX_DONE, the in-flight frame remains associated with the current transaction.

- The next queued payload, empty request, and dedicated SND-NKE frame are prepared as applicable.

- After the expected Meter acknowledgement is accepted, the payload is removed from the queue, FCB advances when required, and in-flight ownership is released.

- On retry, the same in-flight packed frame and transmitted FCB are reused; the frame is not reconstructed or re-encrypted.

# 10. Frame-A Radio Test Mode

## 10.1 Purpose and Boundary

The dedicated SOC_9006_FRAME_TRX firmware configuration provides a controlled two-device RF-driver test using the same WMBus RF driver as the datalink product. It isolates Frame A and RF behavior from normal datalink sessions.

Frame test frame
  -> Format A CRC packing
  -> Manchester / 3-out-of-6 coding
  -> RF TX
  -> RF RX
  -> coding decode
  -> Format A CRC validation
  -> frame header / pattern / sequence validation

## 10.2 Build Configuration

| Property | Value |
| --- | --- |
| Build configuration | SOC_9006_FRAME_TRX |
| RF platform | subg_soc_9006 |
| Linker script | ldscripts/default_9006.ld |
| Compile symbol | WMBUS_FRAME_TEST |
| Datalink include path | protocol/wmbus_datalink |
| Platform interface include path | protocol/wmbus_datalink/platform_intf |

## 10.3 Commands

wm frame_test help
wm frame_test init [role] [mode]
wm frame_test tx [frame_length] [pattern] [count] [interval_ms]
wm frame_test rx [on|off]
wm frame_test status
wm frame_test reset
wm frame_test stop
wm frame_test deinit

## 10.4 Role and Direction Rules

| Role Value | Meaning |
| --- | --- |
| 0 | Other/GW |
| 1 | Meter |

For S1/T1/C1, Meter TX and Other/GW RX are permitted. For S2/T2/C2, both roles may start TX or RX. Both devices must use the same PHY mode.

## 10.5 Test Frame

| Offset | Field | Description |
| --- | --- | --- |
| 0 | L field | frame_length - 1 |
| 1 | Marker | 0xD7 |
| 2 | Version | 0x01 |
| 3 | Pattern ID | 0 to 5 |
| 4..7 | Sequence | Little-endian 32-bit sequence number |
| 8 | Source role | Sender frame-test role |
| 9 | PHY mode | Sender WMBus PHY mode |
| 10..N-1 | Test data | Repeated selected pattern |

## 10.6 Standard T2 Two-Device Procedure

Receiver (Other/GW):

wm frame_test init 0 3
wm frame_test reset
wm frame_test rx on
wm frame_test status

Transmitter (Meter):

wm frame_test init 1 3
wm frame_test reset
wm frame_test tx 32 aa 100 100

A successful local TX normally reports requested = accepted = done and error = rejected = 0. A successful isolated receiver normally reports good equal to the transmitter done count, with error, format, pattern, lost, and sequence_error all zero.

# 11. WMBus Log Parser

## 11.1 Purpose

wmbus_log_parser.py parses UART logs generated with WMBUS_DBG_MSG_FLOW and WMBUS_DBG_MSG_ERROR. It summarizes tagged events, monitors each log source, correlates Meter session triggers with Gateway reception, detects session/source health problems, and prints context around errors.

## 11.2 Build-Time Diagnostic and Timing Instrumentation

The WMBus datalink provides three independent build-time instrumentation flags. WMBUS_LINK_DIAGNOSTICS controls general buffered diagnostics, while WMBUS_LINK_LA_MEASUREMENT and WMBUS_LINK_TIMING_TEST are two alternative timing-measurement methods.

WMBUS_LINK_DIAGNOSTICS
    └─ General diagnostics and buffered log

Timing measurement methods (choose one)
    ├─ WMBUS_LINK_LA_MEASUREMENT : GPIO + Logic Analyzer
    └─ WMBUS_LINK_TIMING_TEST    : MCU tick software timing capture

### 11.2.1 WMBUS_LINK_DIAGNOSTICS

Set this flag to 1 to enable the general WMBus buffered diagnostic path. It enables WMBUS_DBG_MSG_PRINTF(), wm dbg 1-4, Error/Flow/Info/Debug messages, and the deferred GW Rx-to-Tx session log used to produce records such as [GW][SESSION_START], [GW][EXCHANGE], and [GW][SESSION_END]. These logs can be analyzed by wmbus_log_parser.py.

#define WMBUS_DEBUG_MAX_NUM        32
#define WMBUS_DEBUG_MAX_STRING_LEN 160

The buffered message storage therefore consumes approximately 32 x 160 = 5120 bytes of RAM, in addition to control structures and temporary formatting storage. When the flag is 0, the diagnostic macros compile to no-ops, the approximately 5 KB buffered log is not allocated, and wm dbg reports that WMBus diagnostics are disabled at build time. This is the recommended setting for a customer release build.

### 11.2.2 WMBUS_LINK_LA_MEASUREMENT

LA means Logic Analyzer. When enabled, the firmware initializes and toggles dedicated GPIOs at selected GW/Meter ISR, FSM, RX, and TX timing points. An external logic analyzer can therefore measure intervals directly, for example GW RX done ISR to GW Tx API return. This method does not require UART timing logs and generally disturbs the measured CPU path less than extensive software timestamp/log capture.

wmbus_link_gpio_toggle(2);  /* GW SYNC ISR */
wmbus_link_gpio_toggle(4);  /* GW RX done ISR */
wmbus_link_gpio_toggle(6);  /* GW Tx API return */

This mode repurposes and reconfigures multiple GPIOs and is therefore not intended for normal customer pin configuration. GPIO toggling still has a small execution cost. When LA measurement is enabled, the deferred GW Rx-to-Tx buffered log is disabled to avoid adding diagnostic work to the timing path.

### 11.2.3 WMBUS_LINK_TIMING_TEST

This flag enables MCU-tick software timing capture. The implementation records timestamps at key points such as SYNC ISR, Early-RX, RX ISR, FSM processing, parsing, packing, Tx API, radio configuration, and TX done. It also enables the runtime timing commands:

wm timing on
wm timing off
wm timing dump
wm timing status

Two levels of enable are required: WMBUS_LINK_TIMING_TEST=1 must be selected at compile time so that the timing code and CLI exist, and wm timing on must be issued at runtime before valid captures are recorded. This method is convenient for breaking a turnaround interval into internal software stages, but wise_tick_get_counter(), callbacks, and capture storage add a small measurement overhead.

### 11.2.4 Build Combinations

The two timing methods are mutually exclusive. WMBUS_LINK_LA_MEASUREMENT=1 and WMBUS_LINK_TIMING_TEST=1 must not be enabled together. In addition, WMBUS_LINK_TIMING_TEST=1 requires WMBUS_LINK_DIAGNOSTICS=1 because timing results depend on the diagnostic output path.

| Use Case | DIAGNOSTICS | LA | TIMING |
| --- | --- | --- | --- |
| Customer release build | 0 | 0 | 0 |
| Long-term UART log test | 1 | 0 | 0 |
| Software timing sequence analysis | 1 | 0 | 1 |
| Logic Analyzer precision measurement | 0 | 1 | 0 |
| LA with partial general diagnostics | 1 | 1 | 0 |

For GW RX done ISR -> Tx API return, use LA measurement when the goal is the closest external hardware-time measurement. Use DIAGNOSTICS=1 with TIMING_TEST=1 when the goal is to decompose the interval into internal software stages. For a production customer release library, disable all three flags.

## 11.3 Basic Usage

python -B .\wmbus_log_parser.py --input gw=gw.log --input meter="meter_*.log"

Individual logs can also be specified:

python -B .\wmbus_log_parser.py --input gw=gw.log --input meter=meter_80000001.log --input meter=meter_40000001.log

## 11.4 Important Options

| Option | Purpose / Default Behavior |
| --- | --- |
| --context N | Context lines around reported errors. |
| --meter-trigger-period-ms | Expected Meter trigger period; documented default 10000 ms. |
| --meter-trigger-tolerance-ms | Allowed trigger timing tolerance; documented default 3000 ms. |
| --meter-session-idle-timeout-ms | Active-session silence limit; default is 2x trigger period. |
| --session-failure-streak-limit | Consecutive failed sessions reported as error; default 3. |
| --gw-rx-match-window-ms | Window for matching Meter trigger to GW SESSION_START; default 1000 ms. |
| --gw-missed-trigger-limit | Consecutive unmatched Meter triggers reported as error; default 3. |
| --source-silence-timeout-ms | Source timestamp lag limit; default 3x trigger period. |
| --json report.json | Write machine-readable JSON report. |

## 11.5 Diagnostic Codes

| Level | Code | Meaning |
| --- | --- | --- |
| ERR | TRIGGER_STOPPED | Consecutive Meter SESSION_START interval exceeded the configured period plus tolerance. |
| ERR | SESSION_STUCK | An active Meter session had no liveness activity before the idle timeout. |
| ERR | SESSION_FAILURE_STREAK | Consecutive failed sessions reached the configured threshold. |
| ERR | GW_MISSED_TRIGGER_STREAK | Meter triggers were repeatedly absent from the Gateway log within the match window. |
| ERR | SOURCE_SILENT | One source stopped producing timestamped activity while other sources continued. |
| WARN | BOOT_DURING_ACTIVE_SESSION | A Meter boot banner appeared while a Meter session was active. |

## 11.6 Result Classification

| Result | Definition |
| --- | --- |
| PASS | No firmware [ERR], no firmware [WARN], and no parser diagnostic/warning detail. |
| WARN | No error, but firmware [WARN] or recoverable parser warning exists. |
| FAIL | At least one firmware [ERR] or parser-generated ERR diagnostic exists. |

## 11.7 Expected Tagged Events

[GW][SESSION_START] ...
[GW][EXCHANGE] ...
[GW][SESSION_END] ...
[MTR][SESSION_START] ...
[MTR][EXCHANGE] ...
[MTR][SESSION_END] ...

Recoverable examples include ACC_MISMATCH, FCB_DUPLICATE, and REQUEST_TIMEOUT warnings. Firmware [ERR] lines and parser ERR diagnostics cause FAIL.

# 12. Troubleshooting

| Symptom / Message | Likely Interpretation | Recommended Check |
| --- | --- | --- |
| [SEC][RX_MISMATCH] | Observed application Security Mode does not match provisioned policy. | Verify Meter wm sec setting and the GW whitelist sec value. Do not switch mode automatically in response to the frame. |
| Unknown Meter rejected | Fixed/admission-controlled policy or no writable/capacity storage. | Check whitelist mode, admission_ctrl, storage ownership, and capacity. |
| Mode 0 frame rejected | GW entry was not explicitly provisioned as Mode 0. | Provision security_mode=0 before GW start. |
| ACC_MISMATCH warning | Recoverable access-number mismatch / retry condition. | Inspect session sequence, retry count, FCB/ACC behavior, and eventual SESSION_END result. |
| FCB_DUPLICATE warning | Meter received a duplicate FCB and may resend the last response. | Verify that retries retain the same in-flight frame/FCB and that FCB advances only after accepted response. |
| SESSION_STUCK | No Meter liveness during an active session. | Inspect the context printed by the parser and compare Meter/GW SESSION_START/EXCHANGE/SESSION_END events. |
| GW_MISSED_TRIGGER_STREAK | Meter started sessions that were not correlated with GW reception. | Check RF reception, timestamps, device ID, match window, and GW log continuity. |
| SOURCE_SILENT | One UART/log source stopped while others continued. | Check UART capture, device reset/connection, and timestamp continuity. |
| Frame test RX good < TX done | Remote receiver did not accept every transmitted frame. | Inspect RX error/format/pattern/lost/sequence_error counters and RSSI. |
| Whitelist update rejected after start | Structural reconfiguration is intentionally prohibited. | Reset, configure whitelist completely, verify with wm wl list, then start again. |

# 13. CLI Quick Reference

| Command | Purpose |
| --- | --- |
| wm dbg 4 | Enable the documented debug level used in verification examples. |
| wm role 0 / 1 | Select GW / Meter role in normal verification examples. |
| wm mode 3 | Select T2 in the documented mode numbering. |
| wm setinfo ... | Set local WMBus device information. |
| wm sec 0¦5¦7 | Set Meter application Security Mode. |
| wm key <16 hex bytes> | Override shared test key; use identical value on both sides for Modes 5/7. |
| wm gwonly 1 <GW_ID> | Configure the Meter example to accept/use the specified Gateway. |
| wm wl init fix¦dyn [capacity] | Create temporary fixed or dynamic GW whitelist. |
| wm wl add ... [security_mode] | Provision a Meter; omitted mode selects 7. |
| wm wl del [id] | Delete from writable whitelist before start. |
| wm wl clear | Clear whitelist according to ownership rules before start. |
| wm wl free | Free temporary heap-based whitelist before start. |
| wm wl list | Display whitelist configuration and runtime information. |
| wm adm [0¦1] | Read/set unknown-Meter admission control. |
| wm wr ... | Queue application payload according to role-specific syntax shown in the verification procedures. |
| wm start | Start normal WMBus operation. |
| wm fsm 0 | Trigger the Meter session in the supplied verification procedures. |
| wm earlyrx delay [us] | Set Early-RX sample delay. |
| wm frame_test ... | Operate the dedicated Frame-A radio test firmware. |

# 14. Integration Reference

## 14.1 Principal Whitelist Implementation Points

- wmbus_link_gw_admit_meter()

- wmbus_link_gw_learn_whitelist_entry()

- wmbus_link_gw_whitelist_load_fixed_with_security()

- wmbus_link_gw_whitelist_load_dynamic_buffer()

- wmbus_link_gw_whitelist_init_temporary()

- wmbus_link_gw_whitelist_add_device()

- wmbus_link_gw_whitelist_del_device()

- wmbus_link_gw_whitelist_clear()

- wmbus_link_gw_whitelist_free_temporary()

## 14.2 Principal Low-Latency TX Implementation Points

- wmbus_link_gw_enqueue_payload()

- wmbus_link_create_packet_internal()

- wmbus_link_gw_pack_next_frame()

- wmbus_link_gw_promote_next()

- wmbus_link_gw_finalize_in_flight_tx_frame()

- wmbus_link_GW_next_pre_encryption()

- wmbus_link_gw_prepare_early_null()

## 14.3 Event / Platform Integration

The supplied architecture identifies WMBUS_RX_SYNC_WORD, WMBUS_EARLY_RX_PREP, and WMBUS_RX_DONE as important FSM/event-dispatch points. The platform RF layer owns radio sync notification, Early-RX generation/timer/DLL snapshot handling, and raw/staged packed-frame TX interfaces.

## 14.4 Admission Result Callback

| Result | Meaning |
| --- | --- |
| WMBUS_LINK_GW_ADMISSION_ACCEPTED_ADDED | Meter added to writable whitelist storage. |
| WMBUS_LINK_GW_ADMISSION_REJECTED_UNKNOWN | Unknown Meter rejected by fixed/admission-controlled policy. |
| WMBUS_LINK_GW_ADMISSION_REJECTED_TABLE_FULL | No whitelist capacity remained. |
| WMBUS_LINK_GW_ADMISSION_REJECTED_NO_WRITABLE_STORAGE | Learning requested without writable whitelist storage. |

Callback Meter information is valid only during the callback; the application must copy it if it needs to retain it.

# 15. Operational Rules and Limitations

- Security Mode is provisioned, not negotiated.

- Mode 0 does not provide application-payload encryption or cryptographic authentication.

- Mode 0 and Mode 5 Meters must be explicitly provisioned for Gateway dynamic-enrollment scenarios; automatically learned entries use Mode 7.

- Whitelist structural configuration is completed before Gateway start; reset is required for later structural changes.

- Each Gateway connection owns independent queue and prepared-frame state.

- Queue data is consumed only after the expected acknowledgement is accepted.

- Retry reuses the same in-flight prepared frame and its transmitted FCB.

- Early-RX is an optimization only; complete-frame validation remains mandatory.

- Packed RF bytes must not be sent through the raw DLL encoder a second time.

- The described pre-encryption/pre-packed per-connection pipeline applies to the Gateway TX path, not the Meter TX path.

- Frame-A radio test is a separate test firmware path and does not exercise normal datalink sessions/security/whitelist behavior.

# 16. Glossary

| Term | Definition |
| --- | --- |
| ACC | Access number used in WMBus transport/session processing. |
| ACD | Access Demand bit / indication used in link exchange logic. |
| AFL | Authentication and Fragmentation Layer. |
| APL | Application Layer / application payload region. |
| Candidate | Possible Early-RX GW response selected before RX_DONE but not transmitted until full validation. |
| Commit | Acceptance of a still-valid Early-RX candidate after complete RX validation. |
| DLL | Data Link Layer. |
| DFC | Data Flow Control indication. |
| ELL | Extended Link Layer. |
| Fallback | Normal prepared-frame response path used when Early-RX candidate is absent or invalid. |
| FCB | Frame Count Bit; transaction/retry state must remain consistent. |
| FCV | Frame Count Valid bit. |
| In-flight frame | Prepared frame selected for the current transaction whose result is still pending. |
| LTL | Long Transport Layer header form referenced by the packet-format material. |
| MTR | Meter. |
| Pre-encryption | Historical implementation term for the GW preparation pipeline; exact work depends on Security Mode. |
| Pre-packed | RF frame already processed through WMBus RF packing, including Frame A CRC and line coding. |
| STL | Short Transport Layer header form. |
| TPL | Transport Layer. |
| tDEC / tRM / tRO / tTO / tTxD | FSM timing terms defined in Section 4. |

# 17. Supplied Project References

| Source | Use in This Manual |
| --- | --- |
| wmbus_security_mode_guide.md | Current Security Mode configuration and verification behavior. |
| wmbus_whitelist_policy.md | Current Gateway whitelist/admission/storage rules. |
| wmbus_pre_encryption_early_rx.md | Current Gateway pre-encryption, pre-packed TX, Early-RX, retry, and timing architecture. |
| wmbus_frame_a_radio_test_guide.md | Current Frame-A radio test build, commands, frame, procedure, and statistics. |
| wmbus_log_parser.py | Current parser implementation. |
| wmbus_log_parser_usage.txt | Parser usage, diagnostics, and result interpretation. |
| WMbus_DLL_design_draft.pptx | Early implementation and design background. |
| WMBUS_Introduction.pptx | General WMBus background/reference material. |

## 17.1 External Structural Reference

The document organization follows common software user-manual practice: product overview and prerequisites first, followed by setup/configuration, task-oriented operating procedures, reference material, and troubleshooting. The technical content itself is based on the supplied project sources.

# Appendix A. Verification Checklist

| Check | Expected |
| --- | --- |
| Role / PHY mode | GW and Meter configured for intended test; both use the same PHY mode. |
| Identity | Local device information and target Meter/GW IDs are correct. |
| Whitelist | Target Meter exists or dynamic enrollment is intentionally enabled. |
| Security | Meter local mode matches GW per-Meter whitelist mode for application payloads. |
| Key | Modes 5/7 use identical key material on both devices. |
| Start order | Whitelist structure completed before GW start. |
| Session | Expected SESSION_START / EXCHANGE / SESSION_END sequence appears. |
| Retry | Same in-flight frame/FCB retained until transaction result is known. |
| Parser | Final result is PASS, or WARN/FAIL is understood and investigated. |

# Appendix B. Example Log Analysis Command

python -B .\wmbus_log_parser.py ^
  --input gw=gw.log ^
  --input meter="meter_*.log" ^
  --context 20 ^
  --json report.json

The parser also writes wmbus_log_parser_report.txt in the first input log folder.